GL-009 ThermalIQ — ThermalFluidAnalyzer
Build & Delivery Guide

GreenLang Agents Program | Analytics / Calculator Agent

Version 1.0 | December 18, 2025

Purpose: align AI, software, data, thermal engineering, climate intelligence, and validation teams on the end-to-end build plan for GL-009 (ThermalIQ), including required datasets, mathematics, deterministic compliance rules, integration contracts, and acceptance criteria.

# Document Control

| Item | Value | Item | Value |
| --- | --- | --- | --- |
| Agent ID | GL-009 | Codename | THERMALIQ |
| Agent Name | ThermalFluidAnalyzer | Type | Analytics / Calculator |
| Complexity | Medium | Priority | P1 |

Document owners should keep this file versioned in the engineering repository and require change review for any modifications to input/output contracts or mathematics.

# Table of Contents

Right-click to update field.

Note: In Microsoft Word, right-click the table of contents and choose 'Update Field' after opening.

# 1. Executive Summary

GL-009 (ThermalIQ) is a deterministic thermofluid analytics agent that computes thermal efficiency, exergy destruction, thermodynamic property tables, Sankey visualizations, and heat-transfer-fluid recommendations from plant energy/thermal measurements and equipment metadata.

The defining requirement is zero-hallucination: every numeric output must be reproducible, traceable to validated inputs and versioned thermophysical property datasets, and computed by a shared calculation library (no free-form numeric generation).

Primary deliverable: production-grade shared thermodynamic calculation library + service interface used by the agent and other GreenLang agents.

Core analytics: energy balance + exergy analysis with breakdowns by component and loss mechanism.

Decision support: fluid recommendations for 25+ heat transfer fluids, with SHAP/LIME explainability for the recommendation model and sensitivity analysis for deterministic calculations.

Visualization: automated Sankey diagrams (energy and exergy) generated from validated balances.

Integrations: energy meters and process historians via OPC-UA, historian APIs, and GraphQL service layer.

# 2. Agent Definition and Contract

The definition below is the authoritative contract for scope control, interface design, and validation.

| Field | Specification |
| --- | --- |
| ID | GL-009 |
| Codename / Brand | THERMALIQ |
| Internal Name | ThermalFluidAnalyzer |
| Category | Analytics |
| Mode | Calculator (deterministic numeric engine + structured outputs) |
| Complexity / Priority | Medium / P1 |
| Business Value (program) | $7B (as provided in the agent register) |
| Target Milestone | Q1 2026 |
| Status | Implemented (prototype). This guide covers productionization, integration hardening, and validation. |

## 2.1 Core Capability Statement

ThermalIQ provides a shared calculation library with exergy analysis, SHAP/LIME explainability support for 25+ heat transfer fluids, deterministic compliance, Sankey diagram generation, and comprehensive thermodynamic property calculations.

## 2.2 Inputs, Outputs, and Integrations

| Type | Details |
| --- | --- |
| Inputs | Energy inputs; useful heat output; losses; fluid properties; operating conditions; ambient temperature; equipment specifications. |
| Outputs | Thermal efficiency (%); exergy destruction; Sankey diagrams; fluid recommendations; property tables; explainability reports; uncertainty analysis. |
| Integrations | Energy meters; process data historians; OPC-UA; GraphQL. |

# 3. Scope, Use Cases, and Boundaries

## 3.1 Primary Use Cases

Thermal loop performance: evaluate heat-transfer-fluid loop efficiency (supply/return temperatures, flow, pump power) and quantify losses.

Boiler/heater performance: compute useful heat output vs. fuel/electric input; allocate losses (stack, radiation, blowdown, standby).

Heat recovery evaluation: quantify recovered useful heat and exergy benefit vs. rejected heat streams.

Fluid selection and optimization: recommend alternative heat transfer fluids based on operating range, pumping power, heat transfer performance, safety, and stability.

Sustainability reporting: translate energy and exergy results into energy-saving opportunities and emissions impact (when emissions factors are supplied).

## 3.2 Out-of-Scope (for GL-009 MVP)

Detailed CFD or transient finite-element thermal modeling (use specialized simulation tools instead).

Chemical degradation kinetics modeling beyond basic stability constraints unless the fluid dataset explicitly supports it.

Financial optimization (CAPEX/OPEX) beyond providing input-ready metrics (energy savings, pump power delta, expected efficiency gain).

## 3.3 Operating Modes

Steady-state snapshot (single timestamp or averaged window) — default mode for deterministic reporting.

Time-window analysis (rolling or batch) — supports trend analytics, percentile metrics, and anomaly flags (still deterministic).

# 4. End-to-End Workflow (What the Agent Does)

ThermalIQ should implement the following deterministic pipeline. Each step is testable and auditable.

Ingest inputs from meters/historians/spec sheets (OPC-UA, historian APIs, GraphQL payloads, or uploaded files).

Normalize units and timestamps; validate ranges, sensor quality flags, and completeness.

Resolve fluid identity and retrieve versioned thermophysical property models for the target temperature/pressure range.

Compute energy balance: energy in, useful heat out, losses, and any storage term (if applicable).

Compute exergy balance and exergy destruction (total + breakdown), using defined reference environment (T0, P0).

Generate Sankey datasets (node/link tables) and render energy/exergy Sankey diagrams.

If fluid recommendation requested: evaluate candidate fluids, score feasibility and performance, and generate ranked recommendations.

Generate structured outputs (JSON) + human-readable report, including uncertainty propagation and explainability artifacts.

Persist results with full traceability: input snapshot hashes, property dataset version, library version, and calculation configuration.

# 5. Cross-Team Delivery Plan (Who Does What)

## 5.1 Workstreams

WS1 — Shared Thermodynamic Calculation Library (core math + properties).

WS2 — Data Ingestion, Validation, and Integration (OPC-UA, historians, GraphQL).

WS3 — Agent Orchestration (AI) and Explainability (SHAP/LIME + sensitivity).

WS4 — Visualization and Reporting (Sankey + tables + exports).

WS5 — Quality, Determinism, and Compliance (testing, auditability, security).

## 5.2 RACI Matrix (Major Deliverables)

| Deliverable / Task | Product/PM | Thermal Eng | Thermo/Math | AI/ML | Backend | Data Eng | Integration | Climate Intel | QA/Validation | DevOps/Sec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requirements & acceptance criteria | A | C | C | C | R | C | C | C | R | C |
| Thermophysical property dataset (25+ fluids) + versioning | C | A | R | C | C | R | I | C | C | C |
| Shared calculation library (units, energy, exergy) | C | C | A | C | R | C | I | C | R | C |
| Energy balance + loss allocation models | C | A | R | I | R | C | I | C | R | C |
| Exergy analysis methodology + validation | C | C | A | I | R | C | I | C | R | C |
| Fluid recommendation engine + constraints | C | R | C | A | R | C | I | C | R | C |
| Explainability (SHAP/LIME + sensitivity reporting) | I | C | C | A | R | C | I | C | R | C |
| Uncertainty propagation engine | I | C | A | C | R | C | I | C | R | C |
| OPC-UA + historian connectors | I | C | I | I | C | R | A | I | C | C |
| GraphQL/API contract + schema | C | I | I | C | A | C | R | I | C | C |
| Sankey generation (dataset + rendering) | I | C | C | I | A | C | I | C | R | C |
| Security, audit logs, and deployment | I | I | I | I | C | I | I | I | C | A |

Legend: R = Responsible, A = Accountable, C = Consulted, I = Informed.

# 6. Data Requirements (What Must Be Collected)

## 6.1 Minimum Required Inputs

ThermalIQ requires both measured process signals and static asset metadata. Where a signal is unavailable, the system must either (a) compute it from other measured variables, (b) accept a user-provided estimate with explicit uncertainty, or (c) fail fast with a clear error stating what is missing.

Energy inputs: fuel flow and LHV/HHV or electrical input power to heaters; steam/hot-water supply energy where applicable.

Useful heat output: delivered heat to process (e.g., heat exchanger duty, load-side enthalpy rise).

Losses: stack/flue losses, radiation/convection losses, blowdown, distribution losses, unaccounted losses.

Fluid operating conditions: fluid type, supply/return temperature, pressure, flow rate (mass or volumetric), and loop ΔP where relevant.

Ambient/reference environment: ambient temperature (T0) and (optionally) ambient pressure (P0).

Equipment specs: heater/boiler ratings, pump curves or efficiency, heat exchanger UA/effectiveness, insulation class, etc.

## 6.2 Signal Dictionary (Recommended)

| Category | Signal | Description | Units | Sampling | Source |
| --- | --- | --- | --- | --- | --- |
| Energy In | fuel_flow | Fuel mass/volumetric flow rate to heater/boiler | kg/s or Nm³/h | ≥1/min | Metered / historian |
| Energy In | fuel_LHV | Lower heating value (or HHV) for fuel | MJ/kg | static or daily | Lab / ERP / manual |
| Energy In | electric_power_in | Electrical power to heaters/pumps (where needed) | kW | ≥1/min | Power meter |
| Useful Heat Out | process_duty | Heat delivered to process/load (calculated or measured) | kW | ≥1/min | Calculated or historian |
| Thermal Losses | stack_temp | Flue gas temperature (if stack loss model used) | °C | ≥1/min | Historian |
| Thermal Losses | ambient_temp | Ambient temperature (reference environment) | °C | ≥1/min | Weather/plant sensor |
| Thermal Losses | surface_temp | Equipment surface temperature (radiation/convection loss) | °C | ≥1/min | IR sensor/manual |
| Fluid Loop | fluid_name | Heat transfer fluid identity (must map to property dataset) | enum | static | Asset metadata |
| Fluid Loop | T_supply | Fluid supply temperature | °C | ≥1/min | RTD/TC |
| Fluid Loop | T_return | Fluid return temperature | °C | ≥1/min | RTD/TC |
| Fluid Loop | P_loop | Loop pressure (or pressure at reference point) | kPa | ≥1/min | Pressure transmitter |
| Fluid Loop | flow_rate | Mass flow or volumetric flow (with density for conversion) | kg/s or m³/s | ≥1/min | Flow meter |
| Fluid Loop | deltaP | Differential pressure across loop/heat exchanger (pumping analysis) | kPa | ≥1/min | DP transmitter |
| Equipment | pump_efficiency | Pump efficiency (curve or scalar) | % | static | Spec sheet |
| Equipment | heater_rated_capacity | Rated thermal output of heater/boiler | kW | static | Spec sheet |

## 6.3 Data Quality and Validation Rules

Unit normalization: all inputs converted to internal canonical units (SI) using an explicit unit registry; store original units for traceability.

Time alignment: align signals to a common timestamp or window average; record window definition used for each computation.

Range checks: enforce physical plausibility (e.g., T_supply > T_return for heating duty unless explicitly configured).

Missing data: define per-signal handling (drop, interpolate, carry-forward, or fail). For deterministic reporting, default is fail-fast unless policy allows imputation.

Sensor QA flags: honor historian quality bits; exclude bad-quality points from averages; propagate data quality to uncertainty outputs.

Auditability: persist the exact raw values (or a cryptographic hash) used for each run, plus property dataset version and library version.

## 6.4 Uncertainty Inputs (Required for Uncertainty Analysis Mode)

To produce uncertainty intervals, the system must receive uncertainty for each measured input (e.g., ±% of reading, ± absolute, or a standard deviation). If not provided, ThermalIQ must either use a configurable default library by sensor type or report 'uncertainty unavailable'.

Recommended schema for uncertainties:

{
  "signal": "T_supply",
  "distribution": "normal",
  "sigma": 0.2,
  "units": "degC",
  "source": "sensor_spec"
}

# 7. Thermodynamics and Mathematics Requirements

## 7.1 Canonical Definitions

Reference environment: T0 = ambient temperature (default: measured ambient). P0 = ambient pressure (default: 101.325 kPa unless measured).

Sign conventions: define Q_in > 0 as heat/energy entering the system boundary; Q_out > 0 leaving. Losses are treated as Q_out terms.

Steady-state default: dU/dt = 0 unless time-window mode explicitly models storage.

Fluid states: define state points consistently (supply, return, inlet/outlet of heat exchanger).

## 7.2 Energy Balance and Thermal Efficiency

At the chosen system boundary, the energy balance (steady-state) is:

Q_in + W_in = Q_useful + ΣQ_losses + W_out

Thermal efficiency for a heater/boiler boundary is computed as:

η_thermal = Q_useful / Q_in

Useful heat delivered in a liquid loop may be computed from mass flow and enthalpy rise:

Q_useful = ṁ · (h_return − h_supply)

If an incompressible approximation is configured, enthalpy rise may be approximated as:

Δh ≈ ∫_{T_supply}^{T_return} c_p(T) dT  (often approximated as c_p·ΔT over small ranges)

## 7.3 Thermophysical Property Calculations (25+ Fluids)

The shared property service must provide the following properties as functions of temperature (and pressure when relevant):

Density ρ(T,P)

Specific heat capacity c_p(T,P)

Dynamic viscosity μ(T,P)

Thermal conductivity k(T,P)

Vapor pressure P_vap(T) / boiling point constraints

Freezing point / pour point constraints

Enthalpy h(T,P) and entropy s(T,P) (or consistent proxies for incompressible liquids)

Optional derived values: Prandtl number Pr, Reynolds number Re, Nusselt correlations inputs, bulk modulus (for hydraulic transients if ever needed).

Each fluid entry must include validity ranges (T_min, T_max, P_max), recommended operating limits (e.g., film temperature limit), and a confidence/quality rating. Property values must be generated from: (a) published correlations, (b) vendor datasheets with interpolation, or (c) validated reference tables. The dataset must be versioned and immutable per release.

## 7.4 Exergy Analysis (Core Requirement)

ThermalIQ must compute physical (thermomechanical) exergy and exergy destruction, and report a breakdown by component and loss mechanism.

Flow (specific) physical exergy for a stream (neglecting kinetic/potential terms unless configured):

e = (h − h0) − T0·(s − s0)

Exergy associated with heat transfer at boundary temperature T_b (constant temperature approximation):

X_Q = Q · (1 − T0 / T_b)

For heat transfer across a temperature range, the implementation must support integration or a segmented approximation:

X_Q ≈ Σ [ Q_i · (1 − T0 / T_i) ]  over segments i

Exergy destruction is computed from entropy generation:

X_dest = T0 · S_gen

Equivalent balance form (useful for audits):

X_in + ΣX_Q,in + W_in = X_out + ΣX_Q,out + W_out + X_dest

## 7.5 Component Models (Minimum Set)

ThermalIQ should support a modular set of component models so that exergy destruction and losses can be attributed. Minimum component set for MVP:

Heat source (boiler/heater/electric heater): fuel/electric input → useful heat + losses; optional stack loss model.

Heat exchanger/load: temperature approach; optional UA/effectiveness; exergy destruction due to finite ΔT.

Pumps: electrical input → hydraulic work; efficiency; optional pump power calculation from ΔP and flow.

Distribution losses: pipe losses, insulation losses (can be lumped).

## 7.6 Uncertainty Propagation (Deterministic + Quantified)

Uncertainty analysis must be deterministic given the same random seed and configuration. Two supported approaches are required:

Analytical/linear propagation: compute Jacobian sensitivities and propagate covariance (fast, approximate).

Monte Carlo: sample input distributions; compute output distributions; return confidence intervals and sensitivity measures.

Linear propagation (scalar output y = f(x)):

Var(y) ≈ J · Cov(x) · Jᵀ  where J = [∂y/∂x_i]

Monte Carlo must use: fixed RNG seed per run + recorded seed in the audit log.

# 8. Software Architecture and Engineering Requirements

## 8.1 Reference Architecture

The implementation should follow the modular architecture below (illustrative):

## 8.2 Shared Calculation Library (gl-thermo-core)

ThermalIQ depends on a shared calculation library that must be usable both as an in-process library and as a standalone service. Key engineering requirements:

Pure functions where possible: calculations should be referentially transparent (same inputs → same outputs).

Strict unit handling: all public APIs accept units or use typed quantities; internal canonical SI representation.

Versioned thermophysical datasets: dataset version ID always included in outputs.

Configurable reference environment: T0/P0 and reference states are explicit parameters (not hidden globals).

Numerical stability: handle edge cases (near phase boundaries, extreme viscosities) with clear errors rather than silent extrapolation.

Testability: every public function has unit tests and golden regression cases.

## 8.3 Determinism and Zero-Hallucination Compliance

Because GL-009 is a Calculator agent, numeric correctness and determinism are non-negotiable. The following controls are required:

All numeric values in outputs must originate from deterministic computation or from explicitly provided input values (never from a language model).

If an LLM is used for narrative explanation, it must receive numbers only from the computed results object and must be constrained to cite those values verbatim.

Set LLM randomness to deterministic mode (temperature = 0, top_p = 1) when generating any text that includes numbers.

Every run must produce an audit bundle: input hash, configuration, library version, property dataset version, timestamp, and optional RNG seed.

Hard fail on missing required signals unless an imputation policy is explicitly enabled and disclosed in the report.

Implement 'no silent defaults': any default assumptions (e.g., P0 = 101.325 kPa) must be displayed in the output report and stored in the audit log.

## 8.4 API and Integration Requirements

ThermalIQ must support both industrial data ingestion and modern API consumption.

Required integration patterns:

OPC-UA: map tag names to canonical signal IDs; support quality bits and timestamp retrieval.

Process historians: PI/other historian via existing enterprise connector patterns; pull by tag list and time window.

GraphQL service: provide a stable schema for invoking calculations and fetching results; return strongly typed outputs.

API design rules:

Inputs and outputs must be strongly typed (JSON schema / GraphQL types).

All requests must be idempotent with respect to a 'calculation_id' or deterministic hash.

Support both synchronous (single-shot) and batch (time-window) evaluation modes.

Return both human-readable summaries and machine-readable structured results.

## 8.5 Observability, Security, and Governance

Observability: structured logs, metrics (latency, error rate, missing-signal rate), and traces per calculation.

Security: encryption in transit; RBAC; least privilege for historian/OPC credentials; secrets managed via standard platform tooling.

Data governance: classify input signals (operational vs sensitive); define retention for raw data vs aggregated results; support deletion workflows where required.

Model governance (if ML recommendation is used): dataset lineage, training version, evaluation reports, and drift monitoring.

# 9. Explainability and Traceability

## 9.1 What Must Be Explainable

Fluid recommendation ranking: why Fluid A is recommended over Fluid B for the provided conditions.

Key drivers of thermal efficiency and exergy destruction for the specific run (e.g., which losses dominate).

Uncertainty drivers: which sensors/inputs contribute most to output uncertainty.

## 9.2 SHAP/LIME for Recommendation Models

SHAP and/or LIME must be used to explain the recommendation model when the recommendation engine uses a learned model (e.g., gradient-boosted tree, random forest, or neural net surrogate). Requirements:

Provide per-recommendation feature attribution: top positive/negative contributors to the score.

Use stable feature engineering: avoid features that cannot be reconstructed during inference (training-serving skew).

Persist explainability artifacts (SHAP values / LIME local surrogate coefficients) alongside the recommendation outputs.

Expose explanations in both structured form (JSON) and a human-readable narrative.

## 9.3 Sensitivity Analysis for Deterministic Calculations

For deterministic energy/exergy calculations, explainability should be implemented as sensitivity analysis. Minimum outputs:

Normalized sensitivities for key outputs (η_thermal, X_dest) with respect to key inputs (T_supply, T_return, ṁ, Q_in, etc.).

Tornado-style ranking (top drivers) computed deterministically from partial derivatives or finite differences.

A decomposition of exergy destruction by component (heater, heat exchanger, distribution, pump).

## 9.4 Explainability Report Template (Minimum Sections)

Run summary: calculation ID, time window, asset boundary, reference environment (T0/P0).

Energy balance: totals and residual; dominant losses.

Exergy balance: totals, exergy destruction breakdown, dominant irreversibilities.

Fluid recommendation: ranked list + constraints and trade-offs.

Explainability: SHAP/LIME attributions (if applicable) + sensitivity rankings.

Uncertainty: intervals for key outputs + top uncertainty contributors.

Assumptions and data gaps: explicit list of defaults, imputations, and missing signals.

# 10. Fluid Recommendation Engine (25+ Heat Transfer Fluids)

## 10.1 Recommendation Objective

The recommendation engine proposes alternative heat transfer fluids that are feasible for the operating envelope and improve one or more targets such as thermal efficiency, exergy efficiency, pumping power, safety margin, or environmental profile.

## 10.2 Feasibility Constraints (Hard Rules)

Temperature constraints: T_min_operating < min(T) with safety margin; T_max_operating > max(T) with safety margin; film temperature limits observed.

Phase constraints: vapor pressure at operating temperature must be below system pressure with margin; avoid two-phase operation unless explicitly supported.

Freezing/pour point: must be below minimum ambient/startup temperature with margin.

Compatibility constraints: materials compatibility and seal compatibility flags (if available) must be satisfied or flagged.

Safety constraints: flash point / fire point / toxicity categories must meet site policy; otherwise the fluid is excluded.

## 10.3 Performance Scoring (Soft Optimization)

After feasibility filtering, candidate fluids are scored using a transparent, configurable multi-criteria scoring function.

Example weighted scoring (illustrative):

Score = w1·Δη_thermal + w2·(−ΔPumpPower) + w3·(−RiskIndex) + w4·(−CostIndex) + w5·ΔHeatTransferIndex

Δη_thermal: expected change in thermal efficiency given cp, viscosity, and heat-transfer correlations under the same operating conditions.

ΔPumpPower: change in pumping power from viscosity/density impacts on ΔP and pump efficiency.

RiskIndex: derived from safety (flammability/toxicity) and operational stability margins.

CostIndex: optional if pricing data is provided; otherwise omitted.

HeatTransferIndex: proxy metric derived from k, μ, cp and standard correlations (e.g., influences on h and Nu).

## 10.4 Explainability Requirements for Recommendations

For each recommended fluid: show which constraints it passed, and where its margins sit relative to limits.

Show key property differences at the operating temperature(s): cp, μ, ρ, k, vapor pressure, freezing point.

Provide SHAP/LIME attributions if a learned scoring model is used, otherwise provide deterministic contribution breakdown by criterion.

## 10.5 Climate Intelligence Hooks

The climate intelligence team should provide emissions factors and reporting mappings so that ThermalIQ can translate energy efficiency improvements into avoided emissions metrics. At minimum, define:

Electricity emissions factor (kgCO₂e/kWh) by site/grid and time basis (average vs marginal).

Fuel emissions factors (kgCO₂e per unit fuel) for each fuel type in the dataset.

Reporting boundaries (Scope 1 vs Scope 2) and how thermal energy savings map to ESG reporting.

# 11. Sankey Diagram Generation Requirements

## 11.1 Purpose and Rules

Energy Sankey: visualize energy in → useful heat + losses + storage/residual.

Exergy Sankey: visualize exergy in (fuel/electric + exergy of heat) → useful exergy + exergy destruction + exergy losses.

Balance requirement: Sankey flows must numerically balance within a configurable tolerance; otherwise show a 'residual' flow explicitly.

Deterministic rendering: given the same dataset, the diagram must render identically (stable node ordering and labeling).

## 11.2 Sankey Data Contract (Node/Link Schema)

Sankey datasets must be returned as machine-readable tables for downstream UI rendering. Suggested schema:

{
  "nodes": [
    {"id": "energy_in", "label": "Energy In"},
    {"id": "useful_heat", "label": "Useful Heat"},
    {"id": "loss_stack", "label": "Stack Loss"},
    {"id": "loss_radiation", "label": "Radiation/Convection Loss"},
    {"id": "residual", "label": "Residual/Unaccounted"}
  ],
  "links": [
    {"source": "energy_in", "target": "useful_heat", "value_kw": 720},
    {"source": "energy_in", "target": "loss_stack", "value_kw": 180},
    {"source": "energy_in", "target": "residual", "value_kw": 100}
  ],
  "tolerance_kw": 0.5
}

## 11.3 Example (Illustrative)

Note: values above are illustrative only; production diagrams must be generated from computed balances.

## 11.4 Engineering Implementation Notes

Provide both diagram data (nodes/links) and a rendered image (SVG/PNG) when requested.

Ensure labels include units and basis (kW, MJ/h, etc.) and specify the averaging window.

For batch/time-window mode, provide a Sankey per period or an aggregate Sankey with percentile bands as separate outputs.

# 12. Verification, Validation, and Acceptance Criteria

## 12.1 Verification Strategy

Unit tests: each formula and helper function (units, interpolation, integration) has deterministic unit tests.

Golden regression tests: store canonical input datasets and expected outputs (hash + numeric values) per library version.

Property validation: compare computed properties against trusted reference tables within tolerance across the validity range.

Energy/exergy balance checks: residuals computed and reported; must fall within tolerance for 'pass' status.

Integration tests: OPC-UA tag mapping, historian pull, and GraphQL requests exercised in CI using mocks and at least one staging integration.

## 12.2 Determinism Tests (Mandatory)

Same inputs (including timestamps, dataset versions, and configuration) must yield identical numeric outputs bit-for-bit where feasible (or within a defined floating-point tolerance).

Monte Carlo mode must be reproducible with a recorded RNG seed; repeating the run with the same seed yields identical distributions.

Sankey node/link ordering must be deterministic (stable sorting).

## 12.3 Acceptance Criteria (P1 Release Gate)

All required input signals validated or explicitly flagged as missing with clear error codes.

Thermal efficiency computed and reconciles with energy balance within tolerance.

Exergy destruction computed and reconciles with exergy balance within tolerance.

Property tables generated for the chosen fluid at the operating conditions, including validity warnings when near limits.

Sankey datasets + rendered diagrams generated and balance (or show residual explicitly).

Fluid recommendation returns at least N candidates (configurable) when feasible; otherwise returns an explainable 'no feasible fluids' result.

Explainability report generated (SHAP/LIME for recommendation model or deterministic breakdown), including top drivers and constraints.

Uncertainty analysis returns intervals and top contributors when uncertainty inputs are provided.

Full audit bundle is persisted for every run: input hash, config, library version, property dataset version, and timestamps.

# 13. Deployment and Operations

## 13.1 Packaging and Runtime

Package the shared calculation library as a versioned artifact (e.g., PyPI package, internal artifact registry) and optionally as a microservice.

Containerize the service with pinned dependency versions to ensure reproducibility.

Use explicit configuration management (no environment-dependent defaults that change outputs).

## 13.2 Performance Targets (Guidance)

Single snapshot calculation: sub-second to a few seconds depending on property evaluation and uncertainty mode.

Batch/time-window calculation: bounded by window length; support parallelization by time slice and by fluid candidate in recommendation mode.

Caching: cache property evaluations and interpolations by (fluid_id, T, P, version) to reduce repeated compute.

## 13.3 Operational Monitoring

Monitor missing-signal rates and invalid-data rates per asset (often the root cause of low-quality analytics).

Monitor balance residuals: rising residuals may indicate sensor drift or unmodeled losses.

Track recommendation outcomes and 'no feasible fluids' frequency for dataset gaps.

Capture user feedback and override actions to improve future recommendation logic and dataset coverage.

# Appendix A. Proposed Input/Output Schemas (JSON)

## A.1 Calculation Request (Snapshot)

{
  "calculation_id": "optional-idempotency-key",
  "asset": {
    "site_id": "SITE-001",
    "asset_id": "HTF-LOOP-01",
    "boundary": "heater+loop+load"
  },
  "time_window": {
    "start": "2025-12-18T10:00:00Z",
    "end": "2025-12-18T10:15:00Z",
    "aggregation": "average"
  },
  "reference_environment": {
    "T0_degC": 25.0,
    "P0_kPa": 101.325
  },
  "signals": {
    "fuel_flow": {"value": 0.12, "units": "kg/s"},
    "fuel_LHV": {"value": 42.7, "units": "MJ/kg"},
    "electric_power_in": {"value": 15.0, "units": "kW"},
    "T_supply": {"value": 180.0, "units": "degC"},
    "T_return": {"value": 160.0, "units": "degC"},
    "flow_rate": {"value": 8.0, "units": "kg/s"},
    "ambient_temp": {"value": 25.0, "units": "degC"}
  },
  "equipment": {
    "heater_rated_capacity_kw": 1200,
    "pump_efficiency_pct": 72
  },
  "options": {
    "mode": "steady_state",
    "compute_uncertainty": true,
    "recommend_fluids": true,
    "render_sankey": true
  }
}

## A.2 Calculation Response (Snapshot)

{
  "calculation_id": "resolved-id",
  "status": "ok",
  "audit": {
    "input_hash": "sha256:...",
    "library_version": "gl-thermo-core@1.3.0",
    "property_dataset_version": "htf-dataset@2026.01",
    "timestamp_utc": "2025-12-18T10:16:02Z",
    "rng_seed": 12345
  },
  "results": {
    "thermal_efficiency_pct": 84.2,
    "energy_balance_kw": {
      "energy_in": 1000.0,
      "useful_heat": 842.0,
      "losses": {
        "stack": 90.0,
        "radiation_convection": 40.0,
        "other": 28.0
      },
      "residual": 0.0
    },
    "exergy": {
      "exergy_destruction_kw": 210.0,
      "breakdown_kw": {
        "heater": 120.0,
        "heat_exchanger": 70.0,
        "distribution": 20.0
      }
    },
    "fluid_properties": {
      "fluid": "synthetic_heat_transfer_oil_A",
      "at_conditions": [
        {"T_degC": 180, "rho_kg_m3": 800, "cp_kJ_kgK": 2.2, "mu_mPa_s": 2.5, "k_W_mK": 0.12}
      ],
      "validity": {"within_range": true, "warnings": []}
    },
    "sankey_energy": {"nodes": [], "links": []},
    "fluid_recommendations": [
      {"fluid": "candidate_fluid_1", "score": 0.82, "rationale": "…", "explainability": {"shap": []}}
    ],
    "uncertainty": {
      "thermal_efficiency_pct": {"p05": 82.9, "p50": 84.2, "p95": 85.5},
      "top_contributors": [{"signal": "fuel_flow", "contribution_pct": 35}]
    }
  }
}

# Appendix B. Heat Transfer Fluid Coverage Requirements

ThermalIQ must support 25+ heat transfer fluids. The table below is an example starter list; the final list is owned by Thermal Engineering and Data Engineering and must be backed by validated property sources and explicit validity ranges.

| Fluid / Family | Category | Notes / Constraints |
| --- | --- | --- |
| Water (liquid) | Aqueous | General-purpose; high cp; freezing risk. |
| Water/Steam (IF97) | Water/Steam | Use for boilers/steam systems; requires pressure-dependent properties. |
| Ethylene glycol / water (30–60%) | Aqueous glycols | Freeze protection; higher viscosity. |
| Propylene glycol / water (30–60%) | Aqueous glycols | Lower toxicity; higher viscosity. |
| Brine (NaCl/CaCl2 solutions) | Aqueous salts | Freeze protection; corrosion considerations. |
| Mineral thermal oil (low-temp) | Thermal oil | Moderate temperature loops; vendor-specific. |
| Mineral thermal oil (mid-temp) | Thermal oil | Higher stability range; vendor-specific. |
| Synthetic aromatic (biphenyl/diphenyl oxide) | Synthetic | High-temp; vapor-phase possible; requires careful limits. |
| Synthetic hydrocarbon (PAO) | Synthetic | Good low-temp viscosity; stability varies. |
| Alkylated aromatic | Synthetic | High-temp stability; common in industrial HTF. |
| Silicone oil (dimethyl) | Silicone | Wide temperature range; cost considerations. |
| Silicone oil (phenyl) | Silicone | Higher temp capability; cost considerations. |
| Ester-based HTF | Synthetic ester | Biodegradability options; hydrolysis risk. |
| Glycol ether | Organic | Specialty applications; safety review needed. |
| Methanol | Alcohol | Low-temp; flammability risks. |
| Ethanol | Alcohol | Low-temp; flammability risks. |
| Isopropanol | Alcohol | Low-temp; flammability risks. |
| Ammonia (as refrigerant) | Refrigerant | If refrigeration integration needed; safety constraints. |
| R134a (legacy) | Refrigerant | Only if policy allows; environmental constraints. |
| R1234yf | Refrigerant | Lower GWP; if refrigeration integration needed. |
| CO₂ (R744) | Refrigerant | High pressure; specialized. |
| Air | Gas | Hot air systems; low cp; convection constraints. |
| Nitrogen | Gas | Inert hot gas loops. |
| Molten salt (nitrate/nitrite) | Molten salt | Very high temp; freeze risk; corrosion. |
| Liquid sodium (specialized) | Liquid metal | Very high heat transfer; extreme safety constraints. |
| Thermal paste/PCM (if modeled) | PCM | If storage is included; separate model class. |
| Hydrogen (gas) | Gas | Specialized; safety constraints. |
| Helium (gas) | Gas | Specialized high-temp gas cooling. |
| Engine coolant (generic) | Mixture | Automotive/industrial packaged systems. |
| Deionized water | Aqueous | Electrical systems; conductivity constraints. |

Dataset requirement per fluid: provide ρ, c_p, μ, k, h, s (or consistent proxy), vapor pressure/boiling constraints, freezing/pour point, validity ranges, safety flags, and a citation/source reference in the dataset metadata.

# Appendix C. Glossary

| Term | Definition |
| --- | --- |
| Exergy | Maximum useful work obtainable as a system comes to equilibrium with a reference environment (T0, P0). |
| Exergy destruction | Exergy lost due to irreversibility; equals T0·S_gen. |
| Thermal efficiency | Useful heat output divided by energy input at the defined boundary. |
| Sankey diagram | Flow diagram where link width is proportional to magnitude (energy/exergy flows here). |
| SHAP | SHapley Additive exPlanations; feature attribution method for ML model outputs. |
| LIME | Local Interpretable Model-agnostic Explanations; local surrogate model for interpretability. |
| HTF | Heat transfer fluid. |
| T0 / P0 | Reference environment temperature and pressure used for exergy calculations. |

End of document.