GL-015 INSULSCAN
InsulationAnalysisAgent
Build & Delivery Playbook

Energy Conservation | Monitor | Priority: Low | Stage: Implemented

Target release: Q3 2026
Document version: v1.3
Date: December 26, 2025
Owner: Program Management (GL)

Audience: AI/ML, Software Development, Data Engineering, Thermal/Mechanical Engineering, Climate & Energy Economics, Maintenance/Operations, Security/IT, QA/Validation

# Table of Contents

Note: If viewing in Word, insert or update an automatic table of contents (References -> Table of Contents).

# 1. Executive Summary

INSULSCAN (GL-015) is an agent that quantifies insulation-related heat loss from industrial equipment using radiometric thermal imaging and validated heat-transfer mathematics. It converts thermal observations (IR camera data, surface temperatures, ambient conditions) plus equipment geometry and insulation specifications into actionable outputs: repair priorities, heat-loss quantification, economic insulation thickness, ROI optimization, explainability reports, thermal maps, and maintenance recommendations.

## 1.1 Core value proposition

Zero-hallucination heat-loss calculations: deterministic, physics-based results with full traceability to inputs and formulas.

Comprehensive insulation analysis across 50+ insulation materials with temperature-dependent properties and installation attributes.

Economic thickness optimization: determine the insulation thickness that minimizes total annualized cost (energy loss + insulation cost + O&M).

Operational integration: thermal imaging systems + OPC-UA plant signals + CMMS for work orders and maintenance planning.

Explainability: SHAP/LIME for any predictive/ranking models; transparent physics and sensitivity analysis for all calculations.

## 1.2 Primary use cases

Identify insulation defects (missing, wet, compressed, degraded) and quantify associated heat loss and cost.

Prioritize repairs and retrofits by ROI and risk to production/asset integrity.

Compute economic thickness for new insulation designs or retrofit programs.

Generate audit-grade reports for energy efficiency programs and ESG/climate reporting (site-level aggregation optional).

# 2. Scope, Assumptions, and Non-goals

## 2.1 In scope

Heat-loss calculations for insulated and uninsulated surfaces (pipes, valves, flanges, vessels, ducts, tanks, flat plates).

Thermal imaging ingestion and temperature map generation with emissivity and environmental corrections.

Material properties database (>=50 materials) with temperature dependence where available.

Economic analysis: energy cost, fuel mix, boiler efficiency or electricity COP, discount rate, project life.

Outputs for maintenance planning: ranked repair list, suggested actions, and CMMS/OPC-UA integration.

## 2.2 Assumptions (must be explicit in every report)

Steady-state heat transfer unless the user explicitly runs a transient mode (optional future).

Known or estimable emissivity for observed surfaces; reflected apparent temperature can be measured/estimated.

Geometry is sufficiently described (diameter, length, insulation thickness, surface type) to select correct heat-transfer model.

Ambient conditions at capture time (air temperature, wind speed if outdoor, humidity) are provided or measured.

## 2.3 Non-goals (explicitly not promised)

Predicting future failures without labeled history and validated models.

Replacing certified thermography or pressure-vessel inspections; INSULSCAN is decision support.

Generating proprietary drawings or automatically designing insulation attachments (supports recommendations only).

# 3. Stakeholders and Team Responsibilities

This section defines what each team delivers. Use it as a contract for work planning and acceptance.

## 3.1 Workstreams (RACI)

| Workstream | AI/ML | Data Eng | Software Dev | Thermal/Mech Eng | Climate/Econ |
| --- | --- | --- | --- | --- | --- |
| Thermal image ingestion + corrections | C | R | R | A | C |
| Material DB (50+ materials) | C | R | C | A | C |
| Physics heat-loss engine | C | C | R | A | C |
| Economic thickness + ROI engine | C | C | R | C | A |
| Predictive ranking (optional) | R | C | R | C | C |
| Explainability (SHAP/LIME) + sensitivity | A | C | R | C | C |
| OPC-UA + CMMS integrations | C | C | A | C | C |
| QA/Validation + golden dataset | C | R | R | A | C |
| Security, auditability, compliance | C | C | A | C | C |

Legend: R=Responsible, A=Accountable, C=Consulted.

## 3.2 Detailed responsibilities by team

### AI/ML Team

Define where ML is allowed vs where physics is mandatory (default: physics for heat loss).

Develop optional models for: defect classification from thermograms, severity scoring, or ROI ranking when uncertain inputs exist.

Implement SHAP/LIME explainability for any ML-based outputs; generate per-case feature attributions and global summaries.

Maintain model cards: training data provenance, performance, limitations, drift monitors, and safe operating envelopes.

### Data Engineering Team

Define canonical schemas for all inputs (IR frames, metadata, ambient sensors, geometry, insulation specs, costs).

Build ingestion pipelines from thermal imaging systems, historians, OPC-UA, and manual entry; enforce unit normalization.

Implement data quality checks: missingness, range checks, calibration status, timestamp alignment, outlier detection.

Maintain material properties database (versioned), including provenance and validation tests.

### Software Development Team

Implement the calculation services (heat-loss engine, economic engine) with strict determinism and full audit logging.

Expose APIs (REST/gRPC) for batch and interactive runs; support idempotent runs and reproducible outputs.

Implement integration adapters: OPC-UA read (equipment tags, operating states) and CMMS write/read (work orders).

Build UI/reporting pipeline: thermal maps, annotations, prioritized repair list, downloadable reports.

Ensure performance, reliability, and secure deployment (edge and/or cloud).

### Thermal/Mechanical Engineering Team

Own the heat-transfer model selection, assumptions, and validation against standards/handbooks and field measurements.

Define IR thermography capture SOP: camera settings, emissivity measurement methods, distance/angle constraints.

Specify geometry templates for equipment classes (pipes/valves/flanges/vessels/ducts/flat surfaces).

Review and approve defect taxonomy and recommended maintenance actions.

### Climate Intelligence / Energy Economics Team

Define energy-cost inputs and conversion: fuel/electricity tariffs, marginal cost, emissions factors (optional).

Own economic thickness methodology and ROI/NPV/IRR calculations with discounting and project life assumptions.

Set reporting conventions for energy savings, cost savings, and (optional) CO2e savings.

Provide guardrails on interpretation: uncertainty bounds and sensitivity ranges.

# 4. System Context and Architecture

INSULSCAN is designed as a deterministic calculation core with optional ML assist modules. All outputs must be traceable to raw inputs and versioned reference data.

## 4.1 High-level data flow

1) Thermal Imaging Systems - Radiometric IR frames - Camera metadata (calibration, lens, distance) - Capture timestamps 2) Context Inputs - Ambient temperature, humidity - Wind speed (if applicable) - Operating state from OPC-UA (surface/process temps, flow) - Equipment geometry & insulation specs 3) Processing Layer - IR corrections -> surface temperature map - Segmentation/annotation (optional ML + human-in-loop) - Physics heat-loss engine (deterministic) - Economic thickness + ROI optimization 4) Outputs - Heat loss (W, kWh/yr, fuel units) - Cost and savings estimates - Repair priorities (ranked) - Thermal maps & defect locations - Explainability + sensitivity report 5) Integrations - CMMS work orders & maintenance plans - Data lake / historian storage - Dashboards & audit logs

## 4.2 Components (recommended service decomposition)

Ingestion Service: collects IR imagery + metadata; validates calibration and timestamps; stores raw and processed assets.

Correction Service: applies emissivity, atmospheric, and reflected temperature corrections; outputs temperature maps.

Geometry Service: normalizes equipment geometry; selects model family (cylinder/sphere/plate) and parameters.

Physics Engine: computes heat loss with uncertainty bounds and a full audit trail (inputs, constants, formulas, versions).

Economic Engine: computes economic thickness, ROI, NPV, IRR; supports scenarios and sensitivity analysis.

Explainability Service: generates SHAP/LIME attributions for ML outputs; generates physics sensitivity (partial derivatives / perturbation).

Integration Adapters: OPC-UA reads (tags), CMMS writes (work orders), and optional historian/data lake connectors.

Reporting/UI: thermal maps, recommendations, and exported PDF/DOCX reports.

# 5. Data Requirements (Inputs, Schemas, Units, Quality)

## 5.1 Minimum viable input set (per inspection)

IR camera data: radiometric thermogram(s) with per-pixel temperature or radiance + calibration metadata.

Ambient conditions at capture: air temperature, relative humidity; wind speed (outdoor) or convection regime indicator.

Surface info: emissivity (measured/assumed) and reflected apparent temperature estimate.

Equipment geometry: type (pipe/valve/vessel/plate), dimensions, orientation, exposed length/area, insulation thickness.

Insulation specs: material type, installation condition (new/aged/wet), jacketing type, gaps/damage notes.

Energy economics: energy price (fuel/electric), operating hours, conversion efficiency (boiler, heater, COP).

## 5.2 Strongly recommended inputs (improves accuracy)

Process-side temperature or fluid temperature (from OPC-UA/historian) to validate surface measurements and boundary conditions.

Ambient air velocity near surface (anemometer) or site classification (indoors still air vs outdoors windy).

Surface roughness and jacket material to refine convection correlations and emissivity.

Thermography capture metadata: distance, angle, focus, atmospheric transmission model parameters, lens.

Historical maintenance history and prior inspections for trend analysis (optional).

## 5.3 Canonical schema (example fields)

Define one canonical record per analyzed asset instance per inspection. Below is an example; implement as JSON and version it.

{ "inspection_id": "string", "asset_id": "string", "timestamp_utc": "ISO-8601", "ir_image": { "uri": "string", "format": "radiometric", "camera_model": "string", "calibration_date": "date" }, "capture": { "distance_m": 2.5, "view_angle_deg": 20, "emissivity": 0.90, "reflected_temp_C": 25.0 }, "ambient": { "air_temp_C": 22.0, "rh_pct": 55, "wind_mps": 0.5, "pressure_kPa": 101.3 }, "geometry": { "shape": "cylinder", "diameter_m": 0.2, "length_m": 10.0, "orientation": "horizontal" }, "insulation": { "material_id": "MIN_WOOL_001", "thickness_m": 0.05, "condition": "aged", "jacket": "aluminum" }, "operation": { "process_temp_C": 180.0, "operating_hours_per_year": 8000, "state_tag": "RUN" }, "economics": { "energy_type": "natural_gas", "price_per_kWh": 0.06, "system_efficiency": 0.85, "discount_rate": 0.08, "project_life_years": 10 } }

## 5.4 Data quality gates (must-pass for production)

Units normalized (SI internally). Temperatures stored with explicit unit and reference (C/K).

Timestamp alignment: IR capture timestamp within tolerance of ambient and OPC-UA readings (configurable, e.g., +/- 5 min).

Calibration status: camera calibration date within allowed window; otherwise flag results as low confidence.

Range checks: emissivity in [0.1, 1.0], wind speed >= 0, thickness >= 0, plausible temperature bounds per asset class.

Geometry completeness: required fields present for selected model; otherwise fail-fast with actionable error message.

# 6. Thermal Imaging: Corrections and Thermal Requirements

## 6.1 Thermal capture SOP (engineering requirement)

Capture radiometric images (not just JPEG palettes) to preserve quantitative temperature/radiance data.

Record emissivity method: tape/paint reference, contact measurement, or material look-up with uncertainty.

Minimize reflected sources; record reflected apparent temperature (using crumpled foil method or equivalent).

Record distance, angle, and atmospheric conditions; avoid extreme angles that distort emissivity assumptions.

For outdoor assets, record wind estimate and note direct solar loading; flag solar-affected images.

## 6.2 Correction pipeline (must be documented and versioned)

Emissivity correction: convert measured radiance to true surface temperature given emissivity and reflected temperature.

Atmospheric transmission correction: account for distance, humidity, and temperature effects on IR attenuation.

Spatial alignment: map pixel coordinates to asset regions (manual annotation or automated segmentation).

Uncertainty propagation: track uncertainty from emissivity, reflected temperature, and ambient parameters into final temperature map.

## 6.3 Minimum thermal imaging hardware and metadata requirements

Radiometric output: per-pixel temperature or radiance with metadata export (camera settings and calibration).

Temperature range: must cover expected surface temperatures for the site, with headroom for hot spots.

Accuracy/precision: record vendor stated accuracy and noise-equivalent temperature difference (NETD) per camera.

Spatial resolution: sufficient to resolve insulation defects at typical capture distances (define per site; document minimum).

Capture metadata: distance, angle, focus, and lens; store for correction and reproducibility.

# 7. Heat Loss Calculation Engine (Zero-hallucination)

All heat-loss calculations must be deterministic and reproducible. Every numeric output must be accompanied by its input values, formulas, constants, units, and code version.

## 7.1 Modeling approach

Primary method: heat transfer through insulation (conduction) plus outer-surface losses (convection + radiation).

Model families: flat plate, cylinder (pipe), sphere (vessel endcaps), composite layers (insulation + jacketing).

Boundary conditions: internal fluid/process temperature or inner-surface temperature; external ambient temperature; convection regime.

Optional: handle missing internal temperature by using measured outer-surface temperature and invert for heat flux with validated correlations; must report lower confidence.

## 7.2 Convection correlation selection (engineering-owned)

Determine regime: natural convection, forced convection, or mixed (use wind/air velocity and orientation).

Use standard dimensionless groups (Re, Gr, Ra, Pr, Nu) with temperature-dependent air properties.

Maintain a correlation library per geometry/orientation (horizontal/vertical cylinder, flat plate) and validate it against the golden dataset.

If correlation selection is uncertain, compute a bounded range of h and propagate to heat-loss uncertainty.

## 7.3 Core formulas (overview)

Representative equations (details and selection rules must be implemented in the engineering spec):

Conduction (flat): q = k*A*(T_hot - T_cold)/L

Conduction (cylinder): q = 2*pi*k*L*(T_i - T_o) / ln(r_o/r_i)

Convection: q = h*A*(T_s - T_inf) where h from appropriate correlation (natural/forced)

Radiation: q = epsilon*sigma*A*(T_s^4 - T_sur^4)

Thermal resistance network: q = (T_hot - T_amb) / R_total; R_total = sum(R_layers) + R_conv + R_rad_equiv

## 7.4 Geometry-specific requirements

Pipes: cylinder model with optional fittings (valves/flanges) treated as separate shapes with validated correction factors.

Vessels/tanks: cylinder + heads; allow separate zones (shell, head, nozzles).

Flat surfaces/ducts: plate model with appropriate convection correlations.

Spatial aggregation: compute per-region heat loss and then sum to asset and site totals; keep per-region traceability.

## 7.5 Zero-hallucination controls (mandatory)

No unsupported numeric claims: if an input is missing, either (a) block computation, or (b) compute with explicit default and label as assumed with uncertainty.

Traceability artifact: each run produces a machine-readable calculation trace (inputs, intermediate variables, formulas, constants, versions).

Guardrails: enforce physical plausibility (e.g., heat loss cannot be negative; temperatures consistent with second law).

Reproducibility: given identical inputs and versions, outputs must be stable within defined floating-point tolerances.

# 8. Economic Thickness, ROI, and Optimization

## 8.1 Economic thickness objective

Economic thickness is the insulation thickness that minimizes total annualized cost. Total annualized cost typically includes: (1) annual energy loss cost, (2) annualized insulation capital cost, and (3) maintenance/O&M impacts.

## 8.2 Required economics inputs

Energy price basis: marginal or blended; specify currency and time basis; support multiple tariffs if needed.

Operating hours per year (or duty cycle).

Conversion efficiency: boiler/heater efficiency for fuels, or COP for electric systems.

Capital cost model: insulation material + labor cost as a function of thickness and geometry; include jacketing cost if applicable.

Financial parameters: discount rate, project life, optional escalation rates.

## 8.3 Recommended methods

Compute annual heat loss as function of thickness: Q(th).

Compute annual energy cost: C_energy(th) = Q(th) * cost_per_energy / efficiency.

Compute annualized capital cost using capital recovery factor (CRF): C_cap_annual(th) = CapEx(th) * CRF.

Minimize C_total(th) = C_energy(th) + C_cap_annual(th) + C_OM(th) over feasible thickness range.

Return: optimal thickness, ROI, payback, NPV/IRR (optional), and sensitivity to energy price and discount rate.

## 8.4 Optimization and uncertainty

Optimization must respect constraints: available thicknesses, mechanical clearances, temperature limits, safety regulations.

Provide confidence bounds: propagate uncertainty from emissivity, convection coefficient, and cost inputs.

Perform scenario analysis: low/base/high energy price; varying operating hours; different convection regimes.

# 9. ML Modules and Explainability (SHAP/LIME)

## 9.1 Where ML is appropriate

Thermogram segmentation and region-of-interest identification (human-in-loop recommended for early deployments).

Defect classification (missing insulation, wet insulation, jacketing damage) when training data exists.

Severity scoring or ranking when multiple uncertain inputs exist; ML output must not override physics heat loss.

## 9.2 Explainability requirements

For every ML prediction used in a decision (priority, classification, severity), generate SHAP or LIME explanations.

Store explanations with the run artifacts: feature values, attribution values, model version, and calibration state.

Provide human-readable summaries: top drivers, directionality, and confidence level.

Add physics sensitivity alongside ML explainability: how heat loss changes with emissivity, wind, thickness, and ambient temperature.

## 9.3 Model governance

Model registry with versioning, approvals, rollback plan, and deprecation policy.

Golden dataset and benchmark suite; evaluate per-asset-class metrics (precision/recall for defect labels).

Drift detection: distribution shifts in temperatures, emissivity, camera models, and operating regimes.

# 10. Outputs and Deliverable Artifacts

## 10.1 Core outputs (per asset)

Heat loss: instantaneous (W) and annualized (kWh/yr or fuel units) with uncertainty bounds.

Cost of loss: annual energy cost; potential savings from repair/retrofit; ROI/payback and NPV/IRR (if enabled).

Economic thickness recommendation: current vs optimal thickness; incremental insulation and savings curve.

Repair priority: ranked list with reason codes (high loss, safety, accessibility, quick payback).

Thermal map: annotated thermogram with regions and temperatures; defect localization.

Explainability pack: calculation trace + ML SHAP/LIME + physics sensitivity + assumptions log.

## 10.2 Site-level outputs (optional aggregation)

Top-N repair opportunities by savings and payback.

Total estimated heat-loss reduction potential and cost savings.

Program dashboard for maintenance planning and execution tracking (CMMS status).

# 11. Integrations (Thermal Systems, OPC-UA, CMMS)

## 11.1 Thermal imaging systems

Support common radiometric formats and vendor exports; store raw, corrected, and annotated versions.

Maintain camera inventory and calibration records; tie each image to camera and settings.

## 11.2 OPC-UA integration

Read-only ingestion: equipment operating state, process temperatures, flow rates, and alarms (as available).

Tag mapping: maintain per-site mapping table (asset_id -> OPC-UA node IDs).

Time alignment: fetch nearest readings to capture time; store provenance and interpolation method.

## 11.3 CMMS integration

Create or update work orders for prioritized repairs; include attachments (thermal map, calculation summary).

Maintain linkage between INSULSCAN findings and CMMS work-order IDs for closure tracking.

Ensure idempotency: re-running analysis should not duplicate work orders without explicit intent.

# 12. Engineering, QA, and Validation Plan

## 12.1 Golden dataset

Assemble representative assets across geometry classes and temperature regimes.

Include ground truth where possible: contact thermometry, known insulation thickness/condition, and controlled tests.

Version dataset; include capture SOP and metadata completeness scoring.

## 12.2 Test strategy

Unit tests for every formula and geometry case; include dimensional analysis checks.

Property-based tests for physical plausibility (monotonicity with thickness, sign checks).

Integration tests for end-to-end runs: ingestion -> correction -> calculation -> report -> CMMS/OPC-UA stubs.

Performance tests for batch processing (site scans) and interactive UI latency.

## 12.3 Acceptance criteria (minimum)

Heat-loss engine matches reference calculations within agreed tolerance per asset class (e.g., <= 5-10% where inputs are well-known).

All outputs include an assumptions section and a trace artifact.

Explainability artifacts produced for all ML outputs; physics sensitivity produced for all analyses.

No high-severity defects in security and reliability tests; audit logs complete.

# 13. Deployment, Security, and Operations

## 13.1 Deployment modes

Edge-first (plant network): local processing for images and OPC-UA; sync summaries to cloud.

Cloud-first: central processing with secure upload; suitable when plant IT allows.

Hybrid: correction at edge, economics/reporting in cloud.

## 13.2 Security and compliance requirements

Encrypt data in transit (TLS) and at rest; enforce least-privilege access for integrations.

Audit logging: who ran analyses, inputs used, versions, and any manual overrides.

Data retention policy: raw images may contain sensitive context; store per customer/site policy.

Supply-chain controls: SBOM, dependency scanning, and signed artifacts for deployments.

## 13.3 Observability

Metrics: run success rate, data quality failures, average processing time, and output completeness.

Alerts: missing calibration, drift signals, integration failures (OPC-UA/CMMS), and anomalous outputs.

# 14. Milestones and Deliverables (Target: Q3 2026)

The timeline below is a recommended plan; adjust to site onboarding realities and data readiness.

| Phase | Target window | Key deliverables | Exit criteria |
| --- | --- | --- | --- |
| P0 - Definition & Standards | Jan-Feb 2026 | Engineering spec, schemas, SOP, material DB v0 | Spec reviewed/approved; sample data ingested |
| P1 - MVP Core Engine | Mar-Apr 2026 | Correction pipeline + physics engine + basic report | Golden dataset matches reference calculations |
| P2 - Economics & Optimization | May-Jun 2026 | Economic thickness + ROI module + scenario analysis | End-to-end payback and thickness outputs validated |
| P3 - Integrations & Pilot | Jul-Aug 2026 | OPC-UA + CMMS connectors; pilot at 1-2 sites | Pilot users close work orders; feedback incorporated |
| P4 - Release Hardening | Sep 2026 (Q3 close) | Security, performance, documentation, training | Acceptance criteria met; release sign-off |

# 15. Appendices

## 15.1 Units, constants, and conventions

Use SI units internally (m, kg, s, K). Display units configurable per customer (C/F; W/kW; kWh/MMBtu).

Temperature: use Kelvin for radiation terms (T^4).

Stefan-Boltzmann constant sigma = 5.670374419e-8 W/m^2/K^4.

Air properties: use temperature-dependent properties (k, nu, Pr) from validated correlations or tables; version the source.

## 15.2 Material database fields (minimum)

material_id, name, category (mineral wool, calcium silicate, aerogel, foam glass, etc.)

thermal_conductivity k(T): function or table; temperature range validity

density, max service temperature, water absorption, compressive strength (as available)

install factors: typical aging factor, wet factor, jacket compatibility, cost ranges

provenance: datasheet reference, date, and validation notes

## 15.3 Risk register (starter)

Emissivity uncertainty dominates error: mitigate with SOP and reference targets.

Outdoor convection variability: capture wind and classify regime; provide bounds.

Material k(T) variability by manufacturer: store ranges and source metadata.

Integration complexity (OPC-UA/CMMS): build adapters with site-specific mapping and robust retries.

User trust: prioritize traceability, reproducibility, and clear assumptions in every report.

## 15.4 Calculation trace artifact (machine-readable)

Every run must emit a calculation trace for auditability and reproducibility. Recommended keys:

run_id, inspection_id, asset_id, timestamp_utc

versions: code_commit, material_db_version, correlation_library_version, model_version (if any)

inputs: all raw inputs with units and provenance (IR, ambient, geometry, insulation, economics)

derived: intermediate variables (surface temps, h values, resistances, U-values, Q(th) curve points)

formulas: identifiers for each equation used and parameter values at evaluation time

outputs: heat loss, costs, ROI, economic thickness, rankings

uncertainty: parameter uncertainties and propagated bounds

warnings: missing inputs, defaults applied, low-confidence flags, calibration issues

## 15.5 Standard report template (human-readable)

All exported reports (PDF/DOCX) should follow a consistent structure:

Cover page: site, asset, inspection date/time, camera used, analysis version.

Data provenance: input sources, calibration status, ambient conditions, and any assumptions/defaults.

Thermal results: annotated thermogram, regions of interest, temperature statistics per region.

Heat-loss results: per-region and total heat loss, uncertainty bounds, and comparison to baseline (if available).

Economics: annual energy cost, savings opportunity, economic thickness recommendation, payback/NPV/IRR (as enabled).

Recommendations: prioritized actions, parts/labor notes, and CMMS linkage.

Explainability: SHAP/LIME for ML outputs (if used) and physics sensitivity summary.

Appendix: calculation trace reference ID, formulas used, and configuration details.